AVR  BOARD  COMPONENTS  IDENTIFICATION

| COMPONENT | TYPE |  |  | VALUE | COLOUR CODE ID | POLARITY | POSITION |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MOV 1 & 2 | MOV |  |  | 471K | No | No | - |
| DIODES |  |  |  |  |  |  |  |
| D1 & D2 | Diodes |  |  | 6A10 | No | Yes |  |
| D3- D7 | Diodes |  |  | 1N4001 | No | Yes |  |
| ZENER DIODES |  |  |  |  |  |  |  |
| ZD1 | Zener |  |  | 12 volts | No | Yes |  |
| ZD2 | Zener |  |  | 6 volts | No | Yes |  |
| RESISTORS (No polarity) |  |  |  |  |  |  |  |
| Colours |  |  |  |  | 1 | 2 | 3 |
| R1, 8, 23 |  |  | 4.7 KΩ |  | Yellow | Purple | Red |
| R2, 12, 20 |  |  | 1 Ω |  | Brown | Black | Gold |
| R3, 14, 17 |  |  | 47 KΩ |  | Yellow | Purple | Orange |
| R4, 9, 10 |  |  | 1 KΩ |  | Brown | Black | Red |
| R5, 21 |  |  | 100 KΩ |  | Brown | Black | Yellow |
| R6, 18 |  |  | 220 KΩ |  | Red | Red | Yellow |
| R7 |  |  | 10 Ω |  | Brown | Black | Black |
| R11 |  |  | 10 MΩ |  | Brown | Black | Blue |
| R13, 15 |  |  | 22 KΩ |  | Red | Red | Orange |
| R16 |  |  | 10 KΩ choke |  | No colours, choke type resistor |  |  |
| R19, 22 |  |  | 10 KΩ |  | Brown | Black | Orange |
| R24 |  |  | 1MΩ |  | Brown | Black | Green |
| R25 |  |  | 330 Ω |  | Orange | Orange | Brown |

| COMPONENT | TYPE | VALUE | COLOUR CODE ID | POLARITY | POSITION |
| --- | --- | --- | --- | --- | --- |
| CAPACITORS Polar (P) Type Non Polar (N) Type |  |  |  |  |  |
|  | Type | Value |  | Code |  |
| C1 | P | 100μF, 35 -50 volts |  | - |  |
| C2, C5, C9 | N | 0.1μF, 250 volts |  | 103 |  |
| C3, C6, C8 | N | 0.47μF, 250 volts |  | 473 |  |
| C4, C7 | N | 0.22μF, 50 volts |  | 223 |  |
| Light emitting diode LED1 Can take any of terminal position on board |  |  |  |  |  |
| Projection on case of Unijunction (UJT) Transistor 2N2464 must match the projection in TR1 position on AVR Board Projection on 2N2646 case Projection on TR1 position on AVR board |  |  |  |  |  |
| IC component SC1 can take any position on the AVR board as long as the 6 pins fall into the SC1 position on board |  |  |  |  |  |
| 8 pin IC LM 741must be correctly inserted on the IC1 position on AVR board. The chip mark on LM 741 must be at the position of U mark on the board. See drawing below. |  |  |  |  |  |